Pothole Detection Pipeline — Optimization Report

Camera Capture Format: RAW vs. JPEG

# 1. Objective

The pothole-detection pipeline initially ran at ~15 FPS with a per-frame inference time of ~209 ms using JPEG-captured frames. To reduce latency, the approach explored was to switch camera capture from JPEG to RAW format, removing the JPEG encode/decode step and sending raw pixel data directly to the model.

# 2. RAW Capture Attempt

## 2.1 Configurations Tested

| Format | Resolution | Size | Transfer Method |
| --- | --- | --- | --- |
| RAW RGB565 | VGA (640x480) | 614 KB | Single HTTP send |
| RAW RGB565 | QVGA (320x240) | 153 KB | Single HTTP send / chunked |
| RAW RGB565 | QQVGA (160x120) | 38 KB | Chunked (1–2 KB segments) |

## 2.2 Issues Encountered

- OV3660 sensor does not support RGB888 — "Requested format not supported" (ll_cam error)

- Width/height values overwritten due to a shared header buffer (hdr[16]) — root-caused and fixed

- HTTP response failed with ERR_RESPONSE_HEADERS_MULTIPLE_CONTENT_LENGTH — resolved by removing manual Content-Length header

- Transfer consistently failed with "Body short" — stopped at ~11,900 of 153,000 bytes due to a single-send lwIP buffer overflow

- Chunked transfer of the 38 KB QQVGA frame also aborted early (at 3–5 KB) across all 15 attempts

- A model input-shape mismatch (256 vs. 160) was also found and corrected during re-testing

## 2.3 Root Cause

The ESP32's WiFi/HTTP stack (lwIP) cannot reliably transmit RAW payloads in the 38 KB–614 KB range — transfers broke in every configuration tested. By contrast, JPEG frames at ~25 KB transmitted successfully and consistently. This points to a hardware/network bandwidth limitation, not a software defect in the capture or inference code.

## 2.4 Was RAW Worth Pursuing Further?

Even in an ideal scenario where RAW transmission worked, it would only eliminate the JPEG decode step — measured at just 0.80 ms — against an inference time of roughly 33 ms. The potential gain was negligible relative to the reliability cost, so the RAW approach was abandoned in favor of optimizing the JPEG pipeline directly.

# 3. Decision: Revert to JPEG, Optimize Pipeline

Camera capture was reverted to JPEG (QVGA 320x240), and effort was redirected toward pipeline optimization — model export/format, input resizing, and frame handling — while keeping JPEG as the transport format.

# 4. Final Performance & Bottleneck Report

## 4.1 System Metrics

| Metric | Value | Notes |
| --- | --- | --- |
| Model RAM (peak) | 0.23 MB | AI model load memory |
| Trace RAM (peak) | 80.04 MB | Python peak during run |
| Process RSS | 510.66 MB | Total memory used on laptop |

## 4.2 Frame Rate

| Metric | Value |
| --- | --- |
| Camera FPS (avg) | 28.13 |
| Inference FPS (avg) | 25.95 |

## 4.3 Latency Breakdown (per frame)

| Stage | Avg | Min | Max | Description |
| --- | --- | --- | --- | --- |
| JPEG Decode | 0.80 ms | — | — | JPEG → BGR conversion |
| Preprocess | 0.90 ms | — | — | Resize 320x240 → 160x160 |
| Inference | 33.75 ms | 31.26 ms | 40.69 ms | YOLO11n pothole detection (main cost) |
| Postprocess | 1.93 ms | — | — | Bounding box generation |
| TOTAL | 37.69 ms | 35.01 ms | 44.68 ms | Sum of all stages |

# 5. Comparison: Before vs. After

| Metric | Before (Initial JPEG) | After (Optimized JPEG) |
| --- | --- | --- |
| FPS | ~15 | 28.13 |
| Inference Time | ~209 ms | 33.75 ms (avg) |
| Total Latency | ~209+ ms | 37.69 ms (avg) |

# 6. Conclusion

RAW frame capture and transfer is not viable on this ESP32 hardware — every tested configuration failed at the WiFi/HTTP transport layer, regardless of resolution or transfer strategy. Analysis also showed that even a working RAW path would have saved under 1 ms against a ~33 ms inference cost, making it a low-value optimization target.

The team reverted to JPEG capture and instead optimized the inference pipeline directly. This raised throughput from ~15 FPS / ~209 ms to 28.13 FPS / 37.69 ms average total latency — a stable, smooth pipeline with no dropped frames. Inference remains the dominant cost (~90% of total latency) and is the appropriate target for any further optimization.